BOAT PRO USA

Spring Dewint erization & Launch Guide

As wint er retreats and sailing season appr oaches, pr operly dewint erizing y our yacht is critical t o ensuring safe, r eliable performance on the wat er. This compr ehensiv e guide walks y ou thr ough the syst ematic pr ocess of bringing your v essel b ack

to life af ter wint er storage, fr om initial inspection thr ough successful launch day .

Impor tant:  This guide assumes y our v essel was pr operly wint erized. If wint erization was incomplet e or questionable,

consider having a marine sur veyor inspect the yacht befor e proceeding. Document ev erything with phot os and not es for

insurance and maint enance r ecords.

Pre-Dewinterization Planning

Timing Your Launch

Begin dewint erization 3-4 w eeks befor e your planned launch dat e. This pr ovides adequat e time t o identif y issues, or der p arts,

and schedule pr ofessional ser vices without rushing. Consider local wat er conditions, marina av ailability , and typical spring

weather p atterns in y our r egion.

Week Focus Ar ea Key Activities

Week 1 Assessment & Ext erior Initial inspection, hull cleaning, har dwar e check , cover remov al

Week 2 Systems R ecommissioning Engine w ork, plumbing r estoration, electrical syst ems, mechanical checks

Week 3 Interior & T esting Interior r estoration, syst em tests, safety equipment v erification

Week 4 Final Pr ep & Launch Bottom p aint t ouch-up, rigging, sea trial pr eparation, launch day

Required Tools & Supplies

Marine-grade antifr eeze (pr oper type for y our syst ems)

Professional Y acht Ser vices & Maritime Solutions

Engine oil and filt ers (corr ect viscosity and type)

Fuel stabilizer and fr esh fuel

Battery char ger and multimet er

Impeller r eplacement kit

Zincs (appr opriat e sizes for y our v essel)

Hull cleaner , wax, and detailing supplies

Basic hand t ools, wr enches, scr ewdrivers

Marine sealant and caulking mat erials

Inspection camera or bor escope

Exterior Assessment & Preparation

1

Initial Visual Inspection

Befor e touching any thing, conduct a thor ough walk -around inspection. Look for obvious damage fr om wint er storms, animals,

or storage mishaps. Check for sagging cov ers, standing wat er, ice damage, or any unusual conditions. Document ev erything

with phot ographs, noting dat es and conditions.

2

Cover and T arp R emov al

On a dr y, calm day , carefully r emov e wint er cov ers and tarps. Inspect each cov er for damage, clean accor ding t o manufactur er

instructions, and st ore properly for next season. R emov e any suppor t structur es, checking for pr essur e points or chafe marks

on gelco at or p aint wher e suppor ts contact ed the hull.

Pro Tip:  Use this oppor tunity t o clean and r epair cov ers while they 're accessible. Pr oper st orage o f cov ers extends their

lifesp an significantly .

3

Hull Inspection

Examine the entir e hull for blist ering, cracks, str ess fractur es, or osmosis. P ay special att ention t o the wat erline, thr ough-hulls,

rudder , and k eel. Look for any gelco at crazing or spider cracks that may hav e dev eloped ov er wint er. If you find concer ning

damage, consult a marine sur veyor befor e proceeding.

4

Hull Cleaning & Bott om P aint Assessment

Pressur e wash the hull t o remov e wint er grime and any biological gr owth. Use appr opriat e marine cleaner s for y our hull

material. Inspect bott om p aint for cov erage, adhesion, and effectiv eness. Not e any ar eas needing t ouch-up or complet e

repainting. Check for marine gr owth that may hav e star ted despit e paint pr otection.

5

Deck Har dwar e & Fittings

Inspect all deck har dwar e including cleats, stanchions, railings, winches, and hat ches. Check for loose fittings, corr osion, or

stress cracks in mounting ar eas. R emov e, clean, and lubricat e all deck har dwar e. Test operation o f all moving p arts. Replace

any questionable fast eners or w orn components.

Caution:  Deck har dwar e failur es under lo ad can be catastr ophic. If any fitting shows mov ement, corr osion, or cracking,

rebedding or r eplacement is mandat ory befor e launch.

Engine & Me chanical Systems

1

Engine Comp artment Inspection

Open engine comp artments and check for obvious issues: standing wat er, oil or fuel leaks, r odent damage, loose wiring, or

corrosion. V erify that drain plugs ar e installed (they should hav e been r emov ed for wint er). Check engine mounts for

deterioration or mov ement.

2

Oil and Filt er Ser vice

Change engine oil and filt ers even if changed during wint erization (old oil can accumulat e moistur e over wint er). Use

manufactur er-specified oil grades. Check transmission fluid lev els and condition. Inspect oil for wat er contamination, which

appear s as a milky appearance or sep aration.

3

Cooling S ystem R estoration

If antifr eeze was used in the raw wat er syst em, flush it complet ely befor e engine star t-up. R eplace the raw wat er impeller as

standar d practice (impeller s can tak e a set during st orage and fail quickly). Check all cooling syst em hoses for cracks, bulges,

or det erioration. Tight en all hose clamps. V erify seacock operation and inspect thr ough-hull fittings.

Critical:  Never run an engine with antifr eeze in the cooling syst em. Flush thor oughly with fr esh wat er befor e attempting

to star t.

4

Fuel S ystem Checks

Inspect fuel lines for cracks or det erioration. Check fuel filt ers and r eplace if necessar y. If the tank was lef t partially full,

consider adding fr esh fuel and additional stabilizer . For diesel engines, check for algae gr owth or wat er contamination. T est

fuel tank v ents t o ensur e they 're clear . Verify fuel shut off valves operat e smoothly .

5

Drive System & Running Gear

For inbo ard vessels, check shaf t packing glands or stuffing bo xes. Adjust if necessar y to allow 1-2 dr ops per minut e when

shaft is tur ning. Inspect shaf t zincs and r eplace if mor e than 50% det eriorat ed. For outbo ards and I/O driv es, inspect trim tabs,

outdrives, and low er units for damage. Check and change low er unit gear oil, looking for wat er intrusion or metal p articles.

6

Propeller Inspection

Remov e the pr opeller and inspect for damage, fishing line, or debris. Check the shaf t taper and k eyway for w ear or corr osion.

Inspect the pr op for dings, bent blades, or cracks. Hav e damaged pr ops pr ofessionally r epaired or r eplaced. Apply anti-seize

compound t o the shaf t befor e reinstalling the pr op, and t orque t o manufactur er specifications.

Electrical Systems

1

Battery Inspection & T esting

Inspect b atteries for corr osion, cracks, or bulging cases. Clean t erminals and cable connections with a wir e brush and b aking

soda solution. T est b attery voltage (12.6V+ for fully char ged 12V b atteries). Lo ad test b atteries or hav e them pr ofessionally

tested. R eplace b atteries mor e than 4-5 y ears old or showing w eak per formance. R einstall b atteries with pr oper securing and

cable connections.

2

Electr ical Connection Inspection

Inspect all visible wiring for chafe, corr osion, or r odent damage. Check wir e terminations and connections for tightness and

corrosion. T est all br eakers and fuses. Spray electrical connections with corr osion inhibit or. Verify bonding syst em continuity

with a multimet er.

3

Electr onics & Navigation S ystems

Power up electr onics one syst em at a time, checking for pr oper operation. T est GPS, char tplott er, radar , radios, and fish finder s.

Verify all connections ar e tight and corr osion-fr ee. Updat e software and char ts as needed. T est ant enna connections and

inspect ant enna mounts for wat er intrusion or corr osion.

Pro Tip:  Spring is an ex cellent time t o updat e electr onic char ts and so ftware while the bo at is still on the har d and y ou

have good int ernet connectivity .

4

Lighting S ystems

Test all navigation lights, int erior lights, anchor lights, and deck lighting. R eplace any bur nt bulbs with corr ect replacements

(LED upgrades r educe pow er consumption). Check light fixtur e gask ets and r eplace if det eriorat ed. V erify spr eader lights and

searchlights operat e corr ectly.

Plumbing Systems

1

Fresh W ater System

Thoroughly flush antifr eeze fr om all fr esh wat er lines by running taps until clear wat er flows. Sanitize the fr esh wat er syst em

with a bleach solution (1/4 cup bleach per 15 gallons), let sit for 3-4 hour s, then flush complet ely. Fill fr esh wat er tanks and

test all faucets, show ers, and wat er pumps. Check for leaks thr oughout the syst em.

2

Water Heat er

Remov e the byp ass if installed during wint erization. R econnect wat er heat er to plumbing syst em. Fill the tank slowly , bleeding

air from the syst em. Check all connections for leaks. Once filled, t est the heat er element or heat ex changer operation.

3

Head and Sanitation S ystems

Flush antifr eeze fr om all head syst ems. Lubricat e head pump mechanisms per manufactur er specifications. T est toilet

operation with fr esh wat er. If you hav e a holding tank , flush it thor oughly and t est the macerat or or deck pump-out fitting.

Check all hoses for det erioration or odor issues. T est Y-v alves and thr ough-hull seacocks for smooth operation.

4

Bilge Pumps & S ystems

Test all bilge pumps (aut omatic and manual) for pr oper operation. Clean bilge pump scr eens and check dischar ge thr ough-

hulls. V erify float swit ches trigger aut omatic pumps corr ectly. Clean bilges thor oughly , removing any oil, debris, or standing

water. Check for new leaks or sour ces o f wat er intrusion.

5

Through-Hull Fittings

Exercise all seacocks and thr ough-hull v alves multiple times t o ensur e smooth operation. If any v alve is stiff or fr ozen, ser vice

it befor e launch. R eplace any questionable seacocks (br onze seacocks older than 15-20 y ears should be consider ed for

replacement). Apply marine gr ease t o valve stems and thr eads.

Critical Safety It em: Failed thr ough-hull fittings and seacocks ar e a leading cause o f vessel sinkings. Any questionable

fitting must be r eplaced befor e launching.

Interior Systems & Comfort

1

Ventilation & Moistur e Contr ol

Open all hat ches, por ts, and lock ers to air out the bo at. Run dehumidifier s if av ailable and shor e pow er is connect ed. Check

for mold, mildew , or musty odor s indicating moistur e problems. Addr ess any v entilation or wat er intrusion issues befor e the

season star ts.

2

HVAC Systems

Test air conditioning and heating syst ems if equipped. R eplace HV AC filters. Clean condenser coils and check r efrigerant lines

for damage or leaks. V erify condensat e drains ar e clear and functioning pr operly .

3

Galley Appliances

Test st ove, ov en, refrigerat or, and fr eezer operation. If y ou hav e propane, check the entir e syst em for leaks using so apy wat er

on all connections. R eplace any det eriorat ed pr opane hoses. V erify proper operation o f propane solenoid and alar m syst ems.

Test galley sink and faucet.

4

Interior Cleaning & R estoration

Deep clean all int erior sur faces, upholst ery, and carpets. Clean and tr eat all t eak or w ood sur faces. Check cushions for mold or

mildew . Wash all linens, cur tains, and bedding. Clean and or ganize all lock ers and st orage ar eas. This is an ex cellent time t o

reorganize and declutt er.

Sailing Systems (For Sailboats)

1

Mast & Rigging Inspection

If your mast was unst epped for wint er, have it pr ofessionally inspect ed befor e re-stepping. Check all standing rigging for meat

hooks (br oken wir e strands), corr osion, or fatigue. Inspect tur nbuckles and t oggles. Examine the mast for cracks, p articularly

around spr eader r oots, chainplat es, and any pr evious r epair ar eas. Check all sheav es and blocks for smooth operation.

2

Running Rigging

Inspect all halyar ds, sheets, and lines for chafe or UV damage. R eplace any questionable lines befor e the season. Lubricat e

winches accor ding t o manufactur er specifications (typically annual ser vice). Check all blocks and jammer s for pr oper

operation. V erify that all lines ar e properly labeled and led corr ectly.

3

Sail Inspection

Remov e sails fr om st orage and inspect thor oughly . Look for UV damage, chafe, t orn stit ching, or b atten pock et issues. Check

all reef points and sail har dwar e. Consider pr ofessional sail inspection and ser vice if sails ar e mor e than 5-7 y ears old. T est

furling syst ems for smooth operation. Ensur e battens ar e properly inser ted and secur ed.

4

Autopilot & S teering S ystems

Inspect st eering cables or hy draulic syst ems for w ear or leaks. T est rudder for smooth mov ement and no play . Lubricat e

steering syst em per manufactur er guidelines. T est aut opilot calibration and operation. Check emer gency tiller operation and

stowage.

Safety Equipment & Compliance

Life J ackets: Inspect for t ears, broken straps, or missing har dwar e. Test inflatable PFDs and r earm CO2 car tridges

if needed. Ensur e you hav e appr opriat e sizes for all p assenger s.

Fire Extinguisher s: Verify pressur e gauges r ead in gr een zone. Check for damage or corr osion. Confir m Co ast

Guar d appr oval tags ar e present and legible. R eplace any expir ed units.

Flares: Check expiration dat es and r eplace expir ed pyr otechnics. V erify you hav e requir ed minimum quantities.

Store in wat erproof, accessible location.

First Aid Kit:  Replenish used supplies. Check medication expiration dat es. Updat e kit b ased on cruising plans

and cr ew medical needs.

Sound Signals:  Test air hor ns and ensur e backup whistle is accessible. R eplace aer osol hor ns older than 3-4

years.

Navigation Lights:  Verify all r equir ed navigation lights w ork pr operly . Carr y spare bulbs. T est anchor light

operation.

VHF Radio:  Test VHF operation and confir m DSC r egistration is curr ent. V erify handheld b ackup radio is char ged

and functional.

EPIRB/PLB:  Test b attery level and v erify registration is curr ent. Check mounting brack et and activ ation

mechanism.

Throwable Flotation:  Inspect cushion or ring buoy for ser viceability . Ensur e it's readily accessible and not

stowed wher e it can 't be quickly thr own.

Documentation:  Verify vessel r egistration is curr ent. Ensur e insurance documents ar e abo ard. Updat e

emer gency contact infor mation.

Regulat ory Compliance:  Requir ements v ary by v essel size, type, and operating ar ea. V erify your specific r equir ements

with the Co ast Guar d or y our local maritime authority . Offshor e vessels r equir e additional safety equipment including life

rafts, EPIRBs, and o ffshor e flar es.

Final Systems Testing

1

Engine Sea T rial (On the Har d)

With the v essel still suppor ted, conduct an initial engine star t and run. Ensur e adequat e wat er supply for cooling (using a

hose-supplied flushing attachment). S tart the engine and let it war m to operating t emperatur e. Check for unusual noises,

vibrations, or smok e. Verify proper oil pr essur e and t emperatur e gauge r eadings. Check for leaks. Run for 15-20 minut es

minimum.

Impor tant:  Never run an engine without adequat e cooling wat er. Engine damage occur s within seconds o f dry running.

2

Integrat ed Systems T est

With shor e pow er connect ed and/or generat or running, operat e all syst ems simultaneously t o verify no cir cuit ov erloads

occur . Test b attery char ging syst ems. V erify all A C and DC cir cuits function pr operly . Test gr ound fault cir cuit int errupt ers

(GFCIs).

3

Final Inspection W alkthrough

Conduct a compr ehensiv e final inspection using y our original inspection not es. Verify all identified issues hav e been

addressed. Check that all thr ough-hulls below the wat erline ar e in pr oper positions (closed if not needed for launch). Ensur e

all drain plugs ar e installed. V erify battery swit ches ar e properly positioned. Check that all t ools and equipment ar e stowed

and secur ed.

Launch Day Procedures

1

Pre-Launch Checklist

All thr ough-hull seacocks pr operly positioned for operation

All drain plugs installed and tight

Batteries connect ed and swit ches on

Fender s and dock lines pr epared and positioned

Propeller clear o f lines or debris

Engine oil and cooling wat er lev els corr ect

Fire extinguisher s accessible and char ged

VHF radio t ested and set t o Channel 16

Bilge pumps operational

All loose gear and equipment secur ed

2

Immediat e Post-Launch

Once in the wat er, immediat ely check bilges for any wat er intrusion bey ond nor mal w eepage. S tart the engine and v erify

proper wat er flow fr om the cooling syst em exhaust. Check st eering for pr oper operation. V erify electrical syst ems function

normally . Monit or for any unusual sounds, vibrations, or per formance issues.

3

Initial Sea T rial

Plan a shor t initial cruise in pr otected wat ers. Test all syst ems under actual operating conditions. Run engine thr ough full RPM

range. T est st eering at v arious speeds. Deploy and r etriev e anchor . Test all electr onics and navigation equipment. Check bilges

frequently during fir st few hour s aflo at. List en and wat ch for any thing unusual.

Pro Tip:  Keep the fir st sea trial shor t (1-2 hour s) and close t o your marina. This allows y ou to return quickly if any issues

develop and pr ovides easy access t o tools and suppor t if needed.

4

Post Sea T rial Inspection

After returning t o dock , conduct another thor ough inspection. Check bilges for wat er accumulation. Inspect engine

comp artment for leaks. V erify all thr ough-hulls and seacocks r emain dr y. Check b attery char ging syst em per formance. Not e

any issues that r equir e attention befor e ext ended cruising.

Common Issues & Troubleshooting

Problem Possible Causes Solutions

Engine w on't star tDead b atteries, fuel issues, wint erizing fluid

still in syst emCheck b attery voltage, v erify fuel supply , ensur e

cooling syst em is flushed

Excessiv e bilge

waterThrough-hull leak , stuffing bo x adjustment

needed, hose failur eIsolat e sour ce, tight en packing gland, inspect all

through-hulls and hoses

Engine ov erheatingFailed impeller , clogged intak e, ther mostat

failur eReplace impeller , clear intak e screen, check

thermostat operation

Batteries w on't

chargeFailed alt ernator, corr oded connections,

battery failur eTest alt ernator output, clean all connections,

load test b atteries

Water pump not

primingAir in lines, damaged pump diaphragm,

clogged strainerBleed air fr om syst em, r eplace pump

components, clean strainer

Foul wat er odor sHolding tank needs cleaning, hose Flush and sanitize tank , replace sanitation

Problem Possible Causes Solutions

deterioration, impr oper v enting hoses, check v ent lines

Professional Service Recommendations

While many dewint erization tasks ar e within the cap abilities o f experienced bo at owner s, cer tain ser vices should be

performed by qualified marine pr ofessionals:

Bottom P ainting:  Professional application ensur es pr oper sur face pr eparation and cov erage for maximum

effectiv eness

Rigging Inspection:  Certified rigger s should inspect standing rigging ev ery 5-7 y ears or af ter any incident

Engine Ser vices:  Complex engine w ork, inject or ser vice, or turbochar ger maint enance r equir es specialized

knowledge

Electr onics Installation:  Radar , AIS, and complex navigation syst em installations benefit fr om pr ofessional

exper tise

Through-Hull R eplacement:  Below-wat erline thr ough-hull w ork should be done by experienced pr ofessionals

Survey & Inspection:  Pre-pur chase sur veys or insurance sur veys requir e cer tified marine sur veyors

Documentation & Record Keeping

Maintaining compr ehensiv e records of all dewint erization w ork, repairs, and upgrades is essential for sev eral r easons. Detailed

documentation pr ovides b aseline data for futur e maint enance cy cles, helps identif y recurring issues, maintains warranty

validity for major components, and significantly enhances v essel v alue for r esale. Y our r ecords should include dat ed phot os of

all major syst ems, r eceipts for p arts and pr ofessional ser vices, not es on any issues discov ered and their r esolutions, and copies

of service r ecords from marine pr ofessionals.

Consider cr eating a v essel maint enance log that tracks hour s of operation, fuel consumption, oil changes, and all ser vice

performed. Digital logs with cloud b ackup ensur e you nev er lose critical infor mation. Many successful yacht owner s maintain

both p aper logs abo ard the v essel and digital b ackups st ored secur ely online.

Preparing for the Season Ahead

With dewint erization complet e and y our v essel successfully launched, tak e time t o plan y our season. R eview y our cruising

goals, whether that inv olves local day sailing, ext ended cruising t o new destinations, or o ffshor e passages. Ensur e your skill

level mat ches y our ambitions, and consider additional training or pr ofessional deliv ery ser vices for ambitious v oyages bey ond

your experience lev el.

Stock the v essel with appr opriat e provisions, sp are parts, and emer gency equipment for y our int ended use. Cr eate a p assage

planning syst em that accounts for w eather windows, tidal conditions, and fuel/wat er requir ements. Join or consult with local

cruising communities t o lear n about anchorages, marinas, and conditions in ar eas y ou plan t o visit.

Boat Pr o USA Ser vices:  For ext ensiv e cruising plans or yacht deliv ery to distant cruising gr ounds, Bo at Pro USA o ffers

professional yacht deliv ery ser vices with experienced, licensed captains. Our r oute planning exper tise and w eather

routing cap abilities ensur e safe, efficient p assages t o your desir ed destinations. Contact us t o discuss y our seasonal

cruising plans and how our pr ofessional ser vices can suppor t your maritime adv entur es.

BOAT PRO USA

Professional Y acht Deliv ery Ser vices | Maritime Consulting | R oute Planning

This guide is provided for informational purposes and does not replace manufacturer specifications or professional marine services.